# Introduction to Modelica Modeling for Building Systems

A Tutorial Developed for MIT Class 4.421: Space-Conditioning Systems for Low-Carbon Buildings

3/29/2021

By David Blum

# I. Requirements

- Dassault Systemes Dymola (v2020), License, and C Compiler

  - For notes about choosing and setting up the C Compiler, see: https://www.3ds.com/products-services/catia/products/dymola/c-compiler/

- Modelica Buildings Library (v7.0.0)

  - For download, see: https://simulationresearch.lbl.gov/modelica/download.html

  - For notes about setting up the library, see:
https://simulationresearch.lbl.gov/modelica/installLibrary.html

II. Additional Resources

This is going to be a brief introduction.  There is much more to learn about how to use Modelica and Dymola.  I recommend the following resources:

- Dymola Users Guide

- Modelica by Example 
(https://mbe.modelica.university/)

- Buildings Library Users Guide (https://simulationresearch.lbl.gov/modelica/userGuide/index.html)

- Buildings Library Tutorials
(https://simulationresearch.lbl.gov/modelica/training.html)

# Part 1: Modelica Syntax and Dymola Development Environment

Modelica is a language, like C++ or Python.  Therefore, you need to learn its syntax and how it can be used to represent models.  Dymola is the development environment that will help us do that, in addition to helping us organize, compile, and simulate our models, as well as explore the results of those simulations.

Therefore, in this part, you will:

- Write a simple heat transfer model in Modelica

- Check that the model is valid

- Simulate the model

- Explore the results

First, let’s define the physical phenomenon we will model.  Consider a heat transfer problem where a small rock is being cooled by an air stream, as shown in the diagram.  We are interested in simulating the temperature of the rock over time.  This can be described by the differential equation and initial condition below:

ρVcvdTdt=hA(Ta-T)

Tt=0=T0

Where:

T is the rock temperature [K]

T0 is the initial rock temperature [K]

Ta is the air temperature [K]

ρ is the rock density [kg/m3]

V is the rock volume [m3]

cv is the rock specific heat capacity [J/kg-K]

h is the rock-air heat transfer coefficient [W/m2-K]

A is the rock surface area [m2]

1. Open Dymola.  Go to File > Save > SaveAs and save the model file as shown in the Figure below (it will be saved as Rock.mo).  Then, switch to the “Modelica Text” view by clicking on the icon indicated in the top Figure on the next page. Your screen should look like the bottom Figure on the next page.

By default, Dymola will open to the “Diagram View.”  This view is used to configure models using graphical blocks that can be pieced together.  We will explore this later in the tutorial.  For now, we are concentrating on the underlying text-based implementation of models.

Modelica Text View

Diagram View

2. Declare all of the variables in the model as shown in the Figure below.

The syntax of defining a basic model is split into two main sections, one that is used to declare all of the variables in the model and one that is used to define the equations of the model.  The declaration section is above the word “equation” and the equations section is below the word “equation.”

The declaration section is used to give variables names and various characteristics.  Here, parameter means the variable is given a value that does not change over time.  Real means that the value of the variable is a real number, as opposed to an Integer or Boolean (True/False).  Other attributes can be assigned, such as the unit and start value.  Finally, “descriptions” of the variables can be given, which are used later to help us document the model.

Notice that all of our variables are given, except the temperature of the rock.  This is our only variable that will change with time and needs to be calculated using an equation (this is also called a “state” variable).  Notice also that the volume and area parameters are defined as a function of the radius parameter, assuming that our rock is a sphere.

3. Write the equations of the model as shown in the Figure below.

Here, we only have one equation.  It is an ordinary differential equation (ODE).  Modelica syntax naturally accepts the specification of variable derivatives with respect to time using the der() syntax.

Notice we can also write equations in an “acausal” manner.  That is, it does not matter which variables appear on each side of the equation.  In addition, it does not matter the order in which we specify equations, if we had more than one.  All of this makes it easier for us to define systems of differential and algebraic equations (DAEs) without having to first solve them analytically and determine the proper order of assigning values to variables.  Instead, we define them in Modelica, and let the tool (in this case Dymola) solve them for us.

4. Check that the model is valid by using the “Check” command, indicated by a green check mark at the top middle of the screen.  You should see messages at the bottom of your screen as shown in the Figure.

Notice that Dymola has identified that there is 1 unknown variable in our model (T), and that there is 1 equation in the model (our ODE).  Since we have the same number of equations and unknowns, we can solve the problem.  Dymola also runs a number of other checks on the model which are not explicitly listed here.  However, if there were problems with the syntax or problems with how the model is implemented, Dymola would try to detect and report them here, as either Warnings or Errors.  Models with Warnings can still be simulated, while models with Errors will not simulate.

Check Model

5. Go to the Simulation view by clicking on the “Simulation” tab at the top of the screen.  Then, setup the simulation of the model using the “Setup” command at the top middle of the screen.  Configure as shown in the Figure.  Finally, simulate the model using the “Simulate” command at the top of the screen.

In the simulation setup, we are configuring the simulation to run for one hour of simulation time, have an output interval of one second, and utilize the Dassl solver with a tolerance of 1e-6.  Note that the output interval of one second does NOT define our integration timestep.  The Dassl solver is a variable time-step solver, meaning the actual integration timestep will be whatever is needed to successfully solve the equations through time, and will vary depending on how quickly the variables are changing.

Clicking the “Simulate” command actually invokes Dymola to perform multiple processes.  First, it processes the equations into a format in which the variables can be sequentially solved.  It also determines which groups of equations may need to be solved iteratively, called algebraic loops (we do not have that problem in this simple example).  Then, it writes the equations into C code and compiles it.  Finally, it executes the compiled code, using the specified solver to solve (or “integrate”) the model through time.

Simulation Setup

Simulate

6. Explore the results by selecting variables to plot in the Variable Browser to the left of the screen.  You can create new plot windows and plot diagrams by using the commands at the top of the screen in the Plot Options tab, which appears when you select a plot window.  Plot the rock temperature and air temperature over time.  How long does it take for the rock to cool down?

Change the values of the parameters for air temperature and initial rock temperature in the Variable Browser within the Value boxes and simulate again.  Does the cool-down time change?  Now change the values of the parameters for heat transfer coefficient or the radius of the rock.  Does the cool-down time change?

New Diagram

New Plot Window

# Part 2: Connectors, Components, and Graphic Syntax

One of the strengths of Modelica is that it is object-oriented.  This helps us be able to build up libraries of component models which can be pieced together to form larger, custom system models.  There are two additional aspects that help us achieve this capability.  The first is the definition of connectors and the second is graphical annotation.

Therefore, in this part, you will:

- Learn the basic concept of connectors

- Use the graphical syntax to connect component models into a larger system

First, let’s again give ourselves an example problem.  Consider heat flow through a multilayer wall, as shown in the diagram below, and a corresponding resistor-capacitor network model (resistor-capacitor modeling in heat transfer is an analogue to electrical circuit modeling).  We are interested in simulating the temperature of the inside air over time given a time-varying outside air temperature.

In this problem, we could write down the governing system of equations and implement the model in Modelica as we did in Part 1.  However, this would be time-consuming.  Instead, let’s utilize the connector, component modeling, and graphic syntax capabilities of Modelica to not only solve this problem, but give us the ability to easily solve other, similar, problems.

Before doing this, we need to explore the nature of the resistance-capacitor model a bit.  To make things easier, we can simplify the model variables by giving the resistors a resistance, R, and the capacitors a capacitance, C, which can each be calculated from the more specific variables described before.

For a resistor, a constitutive equation relates the heat flow through it to the resistance and temperature different across it:

q=T1-T2R

For a capacitor, a state equation relates the heat flow into the capacitor to the capacitance and time-rate-of-change of the temperature:

C1dT1dt=q

For any node where two or more components (a resistor or capacitor) connect, Kirchoff Current and Voltage Laws (also conservation of energy) say that the net heat flow at the node is zero and the temperature of all components at the node is equal.  That is, energy that leaves one component must flow into its neighbors and at the point it connects to its neighbors the temperature must be the same:

q1+q2-q3=0

Thinking back to our original problem, we have 19 unknown variables (4 R’s, 3 C’s, 5 node temperatures, and 7 component heat flows).  We can assign all of the R’s and C’s based on material properties.  Furthermore, we can assume the outside air temperature is just an input that we define.  Therefore, we have 11 remaining unknown variables; 4 remaining node temperatures and 7 component heat flows.  If we build a generic resistor model using the constitutive equation and instantiate 4 of them, that gives us 4 more equations.  Similarly, if we build a generic capacitor model using the state equation and instantiate 3 of them, that gives us another 3 equations.  Finally, if we could “connect” these components together as shown in the diagram, we could gain 1 more equation per node which says that the sum of heat flows is zero at the node.  Then, we would have 4+3+4=11 equations, which will allow us to solve for all 11 unknown variables!

Why did we go through all of this?  What it tells us is we can individually build generic component models that just define the relationship between the heat flow through the component and the temperature(s) at the interface(s).  Then, we can connect these component models together in virtually any configuration and have enough equations to solve for the whole system!

Now, let’s call the heat flow the “flow” variable and the temperature the “potential” variable for a particular component (you can think of heat flowing through the component while the temperature drives the potential for heat flow).  Then, we could say more generally that at a connection node: “flow” variables sum to zero and “potential” variables are equal.  As it turns out, this pairing of “flow” and “potential” variables is analogous in other domains than just heat transfer.  In particular, electrical circuits, fluid flow, translational kinematics, and rotational kinematics!  This is one of the most powerful concepts that Modelica takes advantage of in order to be able to facilitate the simulation of new systems.  If you’re interested in learning more about this concept of modeling that generalizes across physical domains, it is often referred to as Linear Graph Modeling, and a good discussion is presented from the MIT Mechanical Engineering Department:

- Part 1: http://web.mit.edu/2.14/www/Handouts/OnePorts.pdf

- Part 2: http://web.mit.edu/2.14/www/Handouts/TwoPorts.pdf

Having gone through all of this, let’s build our wall model.

1. Open Dymola.  Go to File > Save > SaveAs and save the model file as shown in the Figure below (it will be saved as Wall.mo).

2. Explore the Modelica Standard Library using the Package Browser on the left side of the screen.  In particular, navigate to the Thermal Resistor component model at Modelica.Thermal.HeatTransfer.Components.ThermalResistor.  Then, click and drag it onto the modeling canvas.  Then, double-click on the model to configure it with the name “R1” and parameter R=0.05 K/W.

Double-click on the Thermal Resistor model in the Package Browser to open it.  Switch to the Modelica Text view (as we used in Part 1) and notice that the graphical component has a corresponding text implementation.  In fact, notice that the parameter R is defined just as we did in our analytical model in Part 1, and that Dymola used that information to populate the configuration we just used to assign a value of 0.05 in a graphical context.  Try also switching to the “Documentation” view to read any information the model developer may have given about the model, such as modeling approach, major assumptions, and typical usage.  Finally, switch to the “Icon” view to see how the graphic content is constructed.

After you’ve finished exploring, switch back to the “Diagram” view of our Wall model to continue building.

Modelica Text View

Documentation View

Diagram View

Icon View

3. Add a second resistor to the Wall model by clicking and dragging a second instance of Modelica.Thermal.HeatTransfer.Components.ThermalResistor to the right of R1.  Then, configure it to have the name “R2” and parameter R=0.1 K/W.  Then, click the right heat port of R1 and hold while dragging your mouse to the left heat port of R2.  Then, let go.  This “connects” the two components together at that heat port.  Continue this process of dragging, dropping, configuring, and connecting components until all resistors and capacitors are added according to the specifications in the table below.  For capacitors, use Modelica.Thermal.HeatTransfer.Components.HeatCapacitor.

| Component Name | Parameter Value |
| --- | --- |
| R1 | R=0.05 K/W |
| R2 | R=0.1 K/W |
| ROut | R=0.01 K/W |
| RIn | R=0.02 K/W |
| C1 | C=1e4 J/K |
| C2 | C=1e5 J/K |
| CIn | C=1e3 J/K |

Once the component models are implemented, switch to the Modelica Text view and explore the corresponding text syntax of the wall model.  Notice the declaration of component models, instead of individual variables, and the uses of the “connect” statements in the equation section.

4. Specify the outside air temperature by using Modelica.Blocks.Sources.Sine and Modelica.Thermal.HeatTransfer.Sources.PrescribedTemperature.  For the sine block, use the parameterization shown on the next page.  Flip the direction of the blocks by selecting on them and using Arrange > Flip Horizontal at the top of the screen (a keyboard shortcut is to press the letter “h” while selected).

Note that the sine block has an interface made of an “output” instead of a “heat port” as the heat transfer models do.  This block outputs a Real signal.  In Modelica, we can incorporate the generation, mathematical manipulation, and usage of Real signals in our system models.  The connection of signals from an output of one component to the input of another is done similarly to connecting the heat ports from before.  Here, the prescribed temperature component model uses a Real signal as an input to specify the value of the temperature at its heat port, which allows us to define the outside temperature node for ROut.  In this case, according to a sinusoidal signal defined by the sine block.  Explore some of the other types of Real signal generation blocks, as well as Boolean signals and available logic blocks.  Note that signals are only for the passage of numeric or Boolean values from one component to another and do not represent anything physical like heat ports.

Arrange

5. Simulate the model for 1 day (86400 seconds) and an output interval of 60 seconds with the Dassl solver and tolerance of 1e-6.  Plot the outside temperature and the inside temperature as shown in the Figure below.

Change the x-axis to hours by right-clicking on the word “Time” > Time Unit > h.  Change the y-axis to C by clicking Setup… > Display Unit > degC.  Export the results in the plot to a .csv file by right-clicking the word “Wall” in the Variable Browser > Export Result > Only Plot Window.  Then, for “Files of type” select “Comma Separated Values”, give the file a name, and save it.  Try opening it in excel or using it in a Python script for post-processing.

Try changing the values of the R and C parameters and explore how it changes the indoor air temperature.  Plot the temperatures of C1 and C2 to understand how the temperature is changing through the wall over time.  Add more layers to the wall, or try adding more sources of heat to the outside, like the sun, or inside, like lights!  For this, you will find the model Modelica.Thermal.HeatTransfer.Sources.PrescribedHeatFlow helpful.

Setup

# Part III: System Modeling

Now that you’ve learned the basics of Modelica syntax, the Dymola environment, and connecting component models to form larger models, let’s build and analyze a simple system model that contains a room and HVAC system for cooling.  In doing so, we can explore the performance of a number of design and control scenarios.

Therefore, in this part, you will:

- Implement a system model that includes a room, fan, and weather data.

- Explore the impact of increased thermal mass on the inside air temperature with and without fan operation.

- Add a simple vapor-compression cooling coil with thermostat control to maintain the room air temperature according to a setpoint.

- Adjust the room air temperature setpoint to simulate a demand-response event and compare the cooling power profile to the case without the demand-response event.

1. Open Dymola.  Load the Modelica Buildings Library by File > Open > Load and browsing to the “package.mo” file within the Buildings Library.  Once loaded, you should see Buildings available in your Package Browser.  Then, go to File > Save > SaveAs and save the model file as “System.mo.”

2. Use the following component models to build the system as shown in the diagram below.  Screenshots are shown for how to parameterize the models.  Note that the parameter values to edit are displayed in black text.  Those in grey text are defaults that are already configured and should be left as they are.

| Model Path | Description/Usage |
| --- | --- |
| Buildings.BoundaryConditions.WeatherData.ReaderTMY3 | Loads weather data from a weather file based on EPW. |
| Buildings.BoundaryConditions.WeatherData.Bus | Allows for the use of weather data throughout the model. |
| Buildings.Air.Systems.SingleZone.VAV.Examples.BaseClasses.Room | Single zone room model with 4 exterior walls, a roof, and a floor. The south wall has a large window. The model implements a detailed heat balance to calculate the inside air temperature, including conduction through exterior walls/roof/floor from the outside (using a 1-D finite difference approximation), solar radiation on exterior surfaces and windows, interior radiation exchange, surface convection based on temperature difference and surface orientation, internal gains and schedules, and infiltration. |
| Buildings.Fluid.Movers.FlowControlled_m_flow | Fan model with mass flowrate as control input signal. |
| Buildings.Fluid.Sensors.TemperatureTwoPort | Temperature sensor for supply air. |
| Buildings.Fluid.Sources.Outside | Ideal boundary from where air can flow into the fan and out of the room. Weather data can be used to specify the conditions of the boundary air through time. |
| Modelica.Blocks.Sources.Constants | Generates a constant signal as output. Used here as a control signal for the fan. |

For the “filNam” parameter, specify the full path to the weather file within the quotations.  There are example weather files within the Buildings library.  Here is an example on my system of how the parameter should look:

Notice that the weather file is a .mos file and not a .epw file.  This is because the data format needs to be in a “Modelica table” format to be read into the model.  The Buildings library provides a program to convert .epw files into .mos files.  The program is located at Buildings/Resources/bin/ConvertWeatherData.jar and instructions can be found in the Documentation section “Adding new weather data” of the model Buildings.BoundaryConditions.WeatherData.ReaderTMY3.

3. Simulate the model from day 120 to 125 with a 60 second output interval using the Dassl solver with a tolerance of 1e-6.  Plot the outside air temperature and inside air temperature as shown in the plot below.

4. Increase the thermal mass in the room by increasing the concrete floor slab thickness from 1” to 6”.  Do this by opening the “room” configuration (double-click on the room model).  Then, click on the grey box to the right of the parameter called “matFlo”, which sets the material layers for the floor construction.  Then, click on the small black arrow next to the “material” parameter and choose Edit Text.  Then, change the line that reads “x=0.025” to “x=0.025*6”.  This is the concrete slab floor thickness in meters.  Click OK on all of the open windows.  Now, simulate the model again.  The results in the plot will update.  Compare with the previous simulation by expanding the previous result in the Variable Browser and plotting the room air temperature variable.

What do you notice about the air temperature with higher thermal mass compared to the temperature with lower thermal mass?

5. Turn on the fan by changing the changing the “k” parameter of the model “con” to 0.5.  This will specify that the fan constantly blows 0.5 kg/s of air into the room.  Since it looks like our outside air temperature is always less than our inside air temperature, this will help us cool down the room.  Simulate the model and look at the updated results.

6. The peak inside air temperature still reaches over 305 K, which is about 32 C and 89 F!  Add a simple vapor-compression cooling model and controls by adding the following component models as shown in the diagram and screenshots below.

| Model Path | Description/Usage |
| --- | --- |
| Buildings.Fluid.HeatExchangers.HeaterCooler_u | An ideal heater or cooler with a control input signal that specifies the fraction of maximum heating or cooling power to apply to the passing fluid. |
| Buildings.Controls.Continuous.LimPID | A PID feedback controller that is used to control the cooling power based on room air temperature setpoint and room air temperature measurement. |
| Modelica.Blocks.Sources.CombiTimeTable | The output signal is specified using a table of values that can be defined at particular simulation times. We will use this to specify the room air temperature setpoint. When connecting to the component “conCoo”, use index [1] in the resulting prompt. |
| Modelica.Blocks.Math.Gain | Multiplies the input signal by a constant value and outputs the result. We will use this to convert the thermal power calculated by the cooler model to electrical power by a constant assumed COP. |
| Modelica.Blocks.Interfaces.RealOutput | Allows the input signal to serve as an output for the whole model. This makes it easier to view results, and also allows the model to be connected to other models. |

Click on the grey box to the right of the parameter “table” to edit the table.  We will just define the output value (22 C) at simulation time 0, and let it be constant for the entire simulation.

Two additional parameters will specify the output to be constant segments between time intervals (instead of interpolating), and to hold the last value specified for the rest of the simulation time (instead of another extrapolation scheme).

7. Simulate the model and plot the room air temperature, outside air temperature, and cooling power.  Use the “New Diagram” command icon to split the plot window between temperature and power measurements.  Then, open the “Setup” window, and set the display unit to deg C.

8. Let’s implement a demand-response event on the 4th day between 1pm and 4pm.  To respond, we want to increase the zone temperature setpoint by 3 C.  Edit the setpoint table to achieve this.  Then, simulate the model again.  Compare the power consumption profile to that without the demand response event.  What do you notice?

Zoom in on the demand response day by using plot Setup and modifying the horizontal range.  You must do this for both subplots.  You can also modify the vertical range for the temperature plot, compare the demand response room air temperature measurement to the previous, and further edit the plot properties as you wish.